
Settings Quick Reference

To reset Defaults, reverse battery until aid vibrates, then replace battery correctly. Alternatively, remove battery for two minutes.

Turn on, hold button:

# Pulses

Settings

/Beeps

1


4 meter

2


2 meter

3


1 meter

4


Half meter

5


8 meter




Advanced Settings

6


1 preset, default

7


2 presets

8


3 presets

9


Chirp audio, default

10


Sweep audio

11


Vibration on when plug in, default

12


Vibration off when plug in

13


Disables remote switch, default

14


Enables remote switch

15


4 meter gap finding

16


2 meter gap finding

17


1 meter gap finding

20


Lock present settings

Enabling Advanced Settings

With the aid off, press and hold the button down for about 30 seconds (until the aid gives a final pulse/beep).

Advanced settings will remain enabled until the aid is reset.

Unlocking aid

Same method as for enabling advanced settings. Alternatively reset the aid.

Note: 

· Only certain types of earphone plugs can be used when the remote switch option is enabled. 

· Modes 15, 16 and 17 can only be selected as the second or third preset.

